- Data Collection

Raw sensor data is recorded on the Raspberry Pi 5 at 10 Hz and saved as a JSON session file, alongside a synchronised AVI video recorded at 30 fps. Two files are sessions/ session_20260314_123947.json and session_20260314_123947.avi Each JSON frame contains:

{

"session_id": "20260314_123947",

"participant": "Hongpeng Fu",

"start_time": "2026-03-14T12:39:47",

"sample_rate_hz": 10,

"frames": [

{

"timestamp": 1773508700.1096,

"fsr": {

"left": 1,

"right": 235

},

"tof": {

"left_mm": 24,

"right_mm": 83

},

"imu_left": {

"ax": -0.9673,

"ay": 0.1234,

"az": 0.0512,

"gx": -3.4046,

"gy": 1.2345,

"gz": -0.5678

},

"imu_right": {

"ax": -0.7627,

"ay": 0.0987,

"az": 0.0345,

"gx": 1.0305,

"gy": 0.8765,

"gz": -1.2345

}

}

],

"end_time": "2026-03-14T12:45:00",

"total_frames": 3000,

"camera_frame_timestamps": [1773508700.1, 1773508700.13, ...]

}

- Feature Extraction

The JSON session file is processed offline to produce two CSV outputs.

frames.csv — Per-frame derived signals

| Column | Source | Description |
| --- | --- | --- |
| t | JSON | Unix timestamp |
| fsr_l, fsr_r | FSR | Raw pressure ADC values (0–1023) |
| tof_l, tof_r | ToF | Shin distance (mm); 8190 = out of range |
| imu_l_ax/ay/az | IMU left | Accelerometer (g) |
| imu_l_gx/gy/gz | IMU left | Gyroscope (°/s) |
| imu_r_ax/ay/az | IMU right | Accelerometer (g) |
| imu_r_gx/gy/gz | IMU right | Gyroscope (°/s) |
| lift_angle_l/r | IMU | Shin elevation angle via complementary filter (°) |
| omega_l/r | IMU | Angular velocity magnitude √(gx²+gy²+gz²) (°/s) |
| jerk_l/r | IMU | Rate of change of acceleration magnitude (g/s) |
| tof_vel_l/r | ToF | Shin velocity via gradient (mm/s) |
| frame_label | System | Phase label: rest / lift_left / hold / lower_right |
| knee_angle | Camera optional | Knee extension angle from YOLOv8-pose (°) |
| shin_angle_vertical | Camera optional | Shin angle vs vertical from YOLOv8-pose (°) |
| trunk_lean | Camera optional | Trunk forward/backward lean (°) |
| ankle_disp_px | Camera optional | Ankle vertical displacement from baseline (px) |
| near_visibility | Camera optional | YOLOv8 keypoint confidence score |

reps.csv — Per-repetition metrics

| Column | Source | Description |
| --- | --- | --- |
| session_id | System | Session identifier |
| rep_id | System | Repetition number within session |
| leg | ToF L/R | Left or right leg |
| duration_s | ToF | Total rep duration: rest→lift→hold→lower→rest (s) |
| lift_duration_s | ToF + IMU | Time from onset to peak distance (s) |
| hold_duration_s | ToF + IMU | Time at peak with ω < 5°/s (s) |
| lower_duration_s | ToF + IMU | Time from peak back to rest (s) |
| peak_tof_mm | ToF | Maximum shin distance during rep (mm) |
| shin_lift_angle_deg | IMU | Peak shin elevation angle (°) |
| peak_angular_velocity | IMU | Maximum angular velocity during rep (°/s) |
| jerk_score | IMU | Mean absolute jerk — lower = smoother (g/s) |
| tof_symmetry | ToF L/R | Left-right peak distance asymmetry (0=symmetric, 1=asymmetric) |
| flagged | System | True if any flag is triggered |
| flag_hold | hold_duration_s | Hold duration < 0.1 s |
| flag_angle | shin_lift_angle_deg | Lift angle < 20° |
| flag_jerk | jerk_score | Jerk above 90th session percentile |

- Output Summary and current dashboard

Seated Knee Extension Attributes (But may change accordingly during the trial stage)

| Attribute | Source | Description |
| --- | --- | --- |
| rep_id | System | Repetition number |
| leg | ToF L/R | Left or right leg |
| duration_s | ToF | Total rep duration (s) |
| lift_duration_s | ToF + IMU | Time from onset to peak (s) |
| hold_duration_s | ToF + IMU | Time stationary at peak (s) |
| lower_duration_s | ToF + IMU | Time from peak back to rest (s) |
| shin_lift_angle_deg | IMU shin | Peak shin elevation angle (°) |
| peak_angular_velocity | IMU shin | Maximum angular velocity (°/s) |
| jerk_score | IMU shin | Movement smoothness — lower is smoother (g/s) |
| fsr_unload | FSR L/R | Thigh unloading magnitude during lift |
| tof_symmetry | ToF L/R | Left-right lift symmetry (0=perfect, 1=fully asymmetric) |
| flagged | System | Rep flagged for review |
| flag_hold | hold_duration_s | Hold duration < 0.1s |
| flag_angle | shin_lift_angle_deg | Lift angle < 20° |
| flag_jerk | jerk_score | Jerk above 90th session percentile |

Seated Hip Flexion Attributes  (But may change accordingly during the trial stage)

| Attribute | Source | Description |
| --- | --- | --- |
| rep_id | System | Repetition number |
| leg | ToF + FSR | Left or right leg |
| duration_s | ToF | Total rep duration (s) |
| lift_duration_s | ToF + IMU | Time from onset to peak (s) |
| hold_duration_s | ToF + IMU | Time stationary at peak (s) |
| lower_duration_s | ToF + IMU | Time from peak back to rest (s) |
| shin_lift_angle_deg | IMU shin | Whole-leg elevation angle proxy (°) |
| peak_angular_velocity | IMU shin | Maximum angular velocity (°/s) |
| jerk_score | IMU shin | Movement smoothness — lower is smoother (g/s) |
| fsr_unload | FSR L/R | Thigh unloading magnitude during lift |
| tof_symmetry | ToF L/R | Left-right lift symmetry (0=perfect, 1=fully asymmetric) |
| flagged | System | Rep flagged for review |
| flag_hold | hold_duration_s | Hold duration < 0.1s |
| flag_angle | shin_lift_angle_deg | Lift angle < 20° |
| flag_jerk | jerk_score | Jerk above 90th session percentile |